KITCHEN & WALKER KW RADIAL DRILLING MACHINE
MAINTENANCE SCHEDULE

Preventive maintenance checklist - workshop use

| Machine: Kitchen & Walker KW Radial Drilling Machine | Asset No.: ____________ | Location: ABU DHABI ____________ | Prepared by: PRAKASH GIRI ____________ |
| --- | --- | --- | --- |
| Serial No.: 2012 08 303 ____________ | Department: ____________ | Issue Date: ____________ | Revision: 00 |

Maintenance Schedule

| Frequency | Maintenance activity | Method / acceptance criteria | Responsibility | Record / Date |
| --- | --- | --- | --- | --- |
| Daily / Before use | Clean machine, table, base and working area; remove swarf/chips. | Machine clean and free from swarf. Do not blow chips towards bearings, slides or electrical equipment. | Operator |  |
| Daily / Before use | Check guards, controls, emergency stop and interlocks (where fitted). | All safety devices secure and operating correctly before use. | Operator |  |
| Daily / Before use | Inspect spindle, chuck/arbor and tool holding condition. | No visible damage, abnormal looseness or insecure tooling. | Operator |  |
| Daily / Before use | Check lubrication/oil levels and visible leaks. | Levels within machine markings; no abnormal leakage. Use only the lubricant specified for the machine. | Operator |  |
| Daily / During use | Observe noise, vibration and temperature. | Stop and report abnormal noise, vibration, overheating or smell. | Operator |  |
| Weekly | Clean and lightly lubricate exposed slides/ways, column and moving surfaces. | Surfaces clean with an even lubricant film; movement smooth. | Operator / Maintenance |  |
| Weekly | Inspect drive belts, pulleys and accessible fasteners. | No cracking, fraying, excessive slack or loose fasteners. Isolate power before inspection. | Maintenance |  |
| Weekly | Check coolant system if fitted. | Coolant level adequate; tank/lines/nozzles clean and flowing; no leaks. | Operator / Maintenance |  |
| Monthly | Inspect electrical cable, plug/isolator, switches and enclosure externally. | No damage, loose fittings, contamination or exposed conductors. Electrical repairs by authorised personnel only. | Maintenance / Electrician |  |
| Monthly | Check spindle/chuck run condition and feed operation. | Smooth operation with no abnormal play, sticking or unusual sound. | Maintenance |  |
| Monthly | Inspect radial arm, column clamping and head travel/clamping. | Movement smooth; clamps hold securely with no unintended movement. | Maintenance |  |
| 3 Monthly | Check machine levelling, foundation/anchor bolts and general fasteners. | Machine stable; bolts/fasteners secure. | Maintenance |  |
| 3 Monthly | Inspect lubrication points and lubrication system. | Lubricant reaches required points; lines/fittings free from blockage or damage. | Maintenance |  |
| 6 Monthly | Inspect drive system and gearbox for wear/leakage; check oil condition where applicable. | No abnormal wear/leaks. Replace oil only to manufacturer specification and site procedure. | Maintenance |  |
| 6 Monthly | Inspect spindle taper, chuck/arbor and accuracy condition. | Clean, undamaged mating surfaces; check runout/alignment to workshop tolerance. | Maintenance |  |
| Annually | Comprehensive mechanical and electrical preventive maintenance inspection. | Inspect safety system, bearings, drive, feed, clamping, lubrication and electrical components. Correct defects before return to service. | Qualified Maintenance |  |
| Annually | Verify machine accuracy and condition after overhaul/major repair. | Record inspection results against approved workshop/manufacturer tolerance. | Qualified Maintenance |  |

Safety note: Before maintenance, isolate the machine from electrical power and apply the site lockout/tagout procedure. Never remove guards or work on moving parts while the machine is energised. Follow the machine manufacturer manual and the site HSE procedure if they specify stricter requirements.

Important: This schedule is a practical generic preventive-maintenance template. Exact lubricant grades, quantities, oil-change intervals, belt settings, electrical test values and accuracy tolerances must be taken from the specific Kitchen & Walker KW machine manual/nameplate and your approved site procedure.

| Maintenance performed by | Date | Checked by | Signature |
| --- | --- | --- | --- |