Lifesaving rules

| Significant hazards |  |
| --- | --- |
|  | - Entanglement with rotating parts - Ejection of workpieces or tooling. - Contact with cutting tools and rotating components. - Flying swarf and metal chips. - Manual handling of heavy components. |

| Additional safety precautions |  |  |  |
| --- | --- | --- | --- |
|  | - When machining abnormal workpieces, a Point of Work (Dynamic) risk assessment MUST be carried out to assess the risks. |  | - Ensure all guarding is in place and any interlocks are working. |
|  | - Rings and jewelry must not be worn or be covered in work areas. |  | - Hearing protection should be worn if the indicators are red. Ensure others in the area are complying. |
|  | - Long and loose hair / Loose articles must be contained or secured. |  | - Where fitted, ensure inbuilt LEV is switched on or use mobile LEV if a lot of smoke or fume is being produced. |

| PPE requirements |  |
| --- | --- |
|  | - Safety glasses - Single piece coveralls (Short sleeves) - Safety footwear |
|  | - Gloves must not be worn near rotating equipment - Cut-resistant gloves must be worn when handling swarf, tooling, and when loading or unloading components onto or from the lathe - Chemical-resistant gloves must be worn when changing coolant and when handling chemicals or oils |

WARNING: Do not operate this machine unless you have received formal training in its safe use, have been assessed and proven competent, and are authorised to operate it through successful completion of a competency assessment.

| Step | Pre-operational safety checks |
| --- | --- |
| 1 | - Ensure the work area, table and floor are clean, dry and free from unnecessary items. |
| 2 | - Check emergency stop and normal controls appear serviceable; do not bypass any safety device. |
| 3 | DC injection brake functional |
| 4 | - Emergency stop functional |
| 5 | - Spindle guard fitted, in good condition, and its interlock functional. |
| 6 | No visible machine damage or oil leaks |

| Step | Housekeeping |
| --- | --- |
| 1 | Switch off the machine and reset all guards to a fully closed position. |
| 2 | Leave the machine in a safe, clean, and tidy state. |

| Step | Operational safety |
| --- | --- |
| 1 | Faulty equipment must NOT be used. Immediately report suspect machinery. |
| 2 | Machine operator MUST NOT leave the machine unattended while operational and MUST always maintain vigilance. |
| 3 | - Set the correct speed and feed before starting. Make speed-range or drive changes only as permitted by the machine manufacturer and with the machine stopped where required. |
| 4 | Avoid letting swarf build up on the tool or job. Stop the machine and remove it using a swarf hook or brush. Wear cut-resistant gloves. |
| 5 | - Stand in a stable position, keep hands clear, and start the spindle using the normal control. |
| 6 | Apply feed smoothly. Do not force the tool. Monitor the cut, workholding, sound, vibration and chip formation |
| 7 | Ensure all applicable guards are in place while operating the machine. |
| 8 | Use a brush, chip hook or suitable tool to remove swarf only when it is safe to do so. Never clear swarf with bare hands |
| 9 | Stop the spindle before measuring, checking the hole, repositioning/clamping the work, changing tools, adjusting the workpiece or leaving the machine unattended |

| Step | Loading, unloading & set-up |
| --- | --- |
| 1 | - With the spindle stopped, position the radial arm and drill head to align the tool with the work. Clamp the arm/head/column as required by the machine design |
| 2 | Mark and locate the drilling position using the approved job method |
| 3 | - Set the correct speed and feed before starting. Make speed-range or drive changes only as permitted by the machine manufacturer and with the machine stopped where required |
| 4 | - Apply feed smoothly. Do not force the tool. Monitor the cut, workholding, sound, vibration and chip formation |
| 5 | Remove the chuck key and all setting tools before starting the spindle |
| 6 | - Position the workpiece on the table/base and secure it using a vice, clamps, fixtures or suitable bolting. Never hold the workpiece by hand. |
| 7 | Confirm the intended spindle speed and feed are suitable for the material, tool diameter and operation |
| 8 | Check machine head, radial arm and column are correctly positioned and clamped before cutting |
| 9 | Arrange coolant, if required, and verify hoses will not contact rotating components |
| 10 | Do not use the machine if abnormal noise, damage, leakage, loose components or unsafe conditions are found. Tag/report the defect according to site procedure |
| 11 | Repeat until the workpiece is running within the required tolerance. |
| 12 | Remove all tools from the bed and slides of the machine. |
| 13 | Remove crane lifting accessories. |